Section V: Schedule of Requirements

6.1 List of Goods & Delivery Schedule - Laboratory Equipment for Higher Education Section (Mattakkuliya & Tangalle) of the University

Unit-: Head Office, Ocean University of Sri Lanka, Mattakkuliya.

| Line-Item No | Description of Goods | Final Destination | Delivery Date |  |
| --- | --- | --- | --- | --- |
|  |  |  | Purchaser’s Allowed Delivery Date | Bidder’s Offered Delivery Date |
|  | - Moisture Analyzer. - Bench-top Centrifuge. - Soxhlet Extraction Unit. - Kjeldahl Degestion& Distillation Unit. - Drying Oven. - Water Bath. - Laboratory Incubator. - Hot Plate with Magnetic Stirrer. - Laboratory Blender / Homogenizer. - High – Speed Homogenizer / Ultra Turrax Type. - Borosilicate Glass Desiccator. - Seive Shaker. - Niskin Water Sampler Bottles. - Rutner Water Sampler. - Conductivity Temperature Depth CTD Profiler. - Acoustic Dopler Current Profiler. - Tide Gauge (Non – Contact Radar System. - Spectrophotometer. - Oven. - Muffle Furnaces. - Distilled Water Plant. - Multiparameter. - Salinometer. - Soxhlet Extraction Unit. - Sedwick Rafter. - Food Processor. - Gas Cylinder (5 Kg). - Light Microscope. - Spectrophotometer. - Muffle Furnaces. - Oven. - CHN Elemental Analizer. - Water Distiller. - Orbital Shaker. - Freezer. - Desiccator. - Laboratory Scale 1. - Laboratory Scale 2. - Trinocular Microscope with 8MP Camera. - Laboratory Oven. - Student Microscope. - Crucibles. - Petri Dish. - Fume Hood. - Glass Spreader. - Dissection Tray. - Refrigerator. - Mixer Grinder. | Main Stores, Crow Island, Mattakkuliya, Colombo – 15. | From the date of Acceptance Maximum 90 days |  |

Name of the Bidder     -:

Address of the Bidder -:

Company Seal -:

Date -:

Section V

6.2 Technical Specification & Compliance - Laboratory Equipment for Higher Education Section (Mattakkuliya & Tangalle) of the Ocean University of Sri Lanka

| Item No. | Component Description | Minimum Specifications | Quantity | Bidder’s Response (Yes/No) |  | If “No” comments on the offer |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Moisture Analyzer | Capacity: 50–100 g; Readability: 0.001 g; Heating: Halogen/Infrared; Temperature Range: ambient–160 °C; Moisture Determination Modes: % Moisture, % Dry Matter; Automatic Calibration; Display: Digital LCD; Housing: Powder-coated steel; Power: 220 V, 50 Hz (Warranty: 3 years Minimum) | 1 |  |  |  |
|  |  | Capacity: 50–100 g; Readability: 0.001 g; Heating: Halogen/Infrared; Temperature Range: ambient–160 °C; Moisture Determination Modes: % Moisture, % Dry Matter; Automatic Calibration; Display: Digital LCD; Housing: Powder-coated steel; Power: 220 V, 50 Hz (Warranty: 3 years Minimum) |  |  |  |  |
| 2 | Bench-top Centrifuge | Rotor Capacity: 6×15 ml or 12×50 ml tubes; Maximum Speed: 6,000 rpm; Maximum RCF: ~4,500 × g; Speed Control: Digital adjustable; Timer: 0–99 min, auto stop; Imbalance Detection: Yes; Noise Level: <60 dB; Power: 220 V, 50 Hz; Material: Steel body with corrosion-resistant rotor, (Warranty: 3 year Minimum) | 1 |  |  |  |
| 3 | Soxhlet Extraction Unit | Positions: 6; Extraction Flasks: 250–500 ml borosilicate glass; Condenser Type: Vertical glass condenser; Heating Mantle: Individual, digital temperature control 50–300 °C; Material: Heat-resistant glass, stainless steel supports; Power: 220 V, 50 Hz; Accessories: Clamps, connectors, silicone tubing; Max Solvent Volume per unit: 500 ml (Warranty: 3 years Minimum) | 1 |  |  |  |
|  |  | Positions: 6; Extraction Flasks: 250–500 ml borosilicate glass; Condenser Type: Vertical glass condenser; Heating Mantle: Individual, digital temperature control 50–300 °C; Material: Heat-resistant glass, stainless steel supports; Power: 220 V, 50 Hz; Accessories: Clamps, connectors, silicone tubing; Max Solvent Volume per unit: 500 ml (Warranty: 3 years Minimum) |  |  |  |  |
| 4 | Kjeldahl Digestion & Distillation Unit | Digestion Block: Corrosion-resistant steel; Heating: Electrical with digital temp control 50–450 °C; Distillation: Borosilicate glass assembly, automatic steam distillation; Sample Capacity: 6–12 per run; Nitrogen Determination Range: 0.1–25 mg; Power: 220 V, 50 Hz; Dimensions- minimum: ~600×400×500 mm; Weight: ~25 kg; Safety: Fume extraction port, over-temp protection; Accessories: Glass tubes, flasks, condensers, (Warranty: 3 year Minimum) | 1 |  |  |  |
| 5 | Drying Oven | Type: Forced convection; Chamber Volume: 50–200 L; Temperature Range: Ambient +5 °C to 250 °C; Temp Accuracy: ±1 °C; Heating: Electrical; Shelves: Stainless steel, removable, adjustable; Construction: Powder-coated steel exterior, stainless steel interior; Control: Digital PID controller; Safety: Over-temp cut-off; Power: 220 V, 50 Hz; Dimensions- minimum:~600×600×800 mm; (Warranty: 3 year Minimum) | 1 |  |  |  |
| 6 | Water Bath |  | 1 |  |  |  |
|  |  | Type: Digital water bath, double-walled; Temperature Range: Ambient to 90 °C; Temperature Stability: ±0.5 °C; Bath Volume: 10–20 L; Material: Stainless steel tank, powder-coated steel exterior; Controls: Digital PID controller; Features: Removable concentric rings; mm; Power: 220 V, 50 Hz (Warranty: 3 year Minimum) |  |  |  |  |
| 7 | Laboratory Incubator |  | 1 |  |  |  |
|  |  | Type: Dry air incubator; Temperature Range: 30–60 °C (±0.5 °C); Control: Digital; Chamber Material: Stainless steel; Shelves: Adjustable, removable, stainless steel; Uniformity: ±1 °C; Safety: Over-temperature alarm; Volume: 40–60 L; Power: 220 V, 50 Hz; Dimensions: (Warranty: 3 year Minimum) |  |  |  |  |
| 8 | Hot Plate with Magnetic Stirrer | Plate Size: 150×150 mm; Stirring Speed: 0–1500 rpm; Heating Range: Room temp–300 °C; Accuracy: ±1 °C; Plate Material: Aluminum or ceramic; Housing: Powder-coated steel; Power: 220 V, 50 Hz; Features: Over-temperature protection, digital display (Warranty: 3 year Minimum) | 5 |  |  |  |
| 9 | Laboratory Blender / Homogenizer | Motor: 500–1500 W; Speed: Variable; Capacity: 500–1000 ml; Blade Material: Stainless steel; Container: Glass or stainless steel; Splash-proof lid; Power: 220 V, 50 Hz Accessories: Blade assembly, jar, lid (Warranty: 3 years Minimum) | 1 |  |  |  |
| 10 | High-Speed Homogenizer / Ultra-Turrax Type |  | 1 |  |  |  |
|  |  | Motor Power: 1–2 kW; Speed Range: 5,000–25,000 rpm; Rotor/Stator: Stainless steel, high precision; Max Sample Volume: 500–1000 ml; Control: Digital display with speed and time settings; Material: Stainless steel body, corrosion-resistant; Power Supply: 220 V, 50 Hz; Accessories: Standard rotor/stator, jar, splash-proof lid; Safety: Overload protection, easy-clean design (Warrenty: 3 year Minimum) |  |  |  |  |
| 11 | Borosilicate Glass Desiccator |  | 1 |  |  |  |
|  |  | With a porcelain plate, Sizes (Inner Diameter): 300mm, Component: Includes a heavy-duty, ground-glass lid for an airtight seal., Included Accessory: An unglazed, perforated porcelain plate (for sample storage), Design: Knob-cover style for easy handling. (Warranty: 3 years Minimum) |  |  |  |  |
| 12 | Selve Shaker | Sieve diameter: 250-300mm | 1 |  |  |  |
|  |  | Measuring range: 0.025-5mm (8-10 sieves with pan) |  |  |  |  |
|  |  | Motor power: ~0.25kW |  |  |  |  |
|  |  | Sound: less than 50dB |  |  |  |  |
|  |  | Amplitude: 3-5mm |  |  |  |  |
|  |  | Speed: 1420rpm |  |  |  |  |
|  |  | Overall dimension: ~450*350*350mm |  |  |  |  |
|  |  | Net weight: 25-30kg |  |  |  |  |
|  |  | Power supply: 220V, 50Hz (Warranty: 3 years Minimum) |  |  |  |
| 13 | Niskin Water Sampling Bottles | Flushing - Free-flushing sampling bottles | 1 |  |  |  |
|  |  | Activate - GO Devil messenger (1000MG, sold separately), individually or serially attached to hydro cable. |  |  |  |  |
|  |  | Closer - Manual by messenger |  |  |  |  |
|  |  | Length - ~25" (633mm) |  |  |  |  |
|  |  | Weight - 2.5kg -3.0kg (Warranty: 3 years Minimum) |  |  |  |  |
| 14 | Ruttner Water Sampler | Volume - 1.0 1.5 L | 1 |  |  |  |
|  |  | Cylinder tube, acrylic - O.D./I. D: 8.5/8.0 cm |  |  |  |  |
|  |  | Length of tube - 24 - 26 cm |  |  |  |  |
|  |  | Total height- 55-60 cm |  |  |  |  |
|  |  | Internal thermometer (non-Mercury) - Range: -10 °C to +60 °C |  |  |  |  |
|  |  | Drop messenger - brass: 200 g |  |  |  |  |
|  |  | Recommended line - Max. Ø2 or Ø3 mm |  |  |  |  |
|  |  | Weight - 1.30-1.5 kg (Warranty: 3 years Minimum) |  |  |  |  |
| 15 | Conductivity Temperature Depth CTD Profiler | Initial Accuracy: - | 1 |  |  |  |
|  |  | Conductivity: ± 0.0005 S/m |  |  |  |  |
|  |  | Temperature: ± 0.005 °C |  |  |  |  |
|  |  | Pressure: Strain-gauge ± 0.1% of full-scale range; Quartz ± 0.02% of full-scale range |  |  |  |  |
|  |  | Total Stability: - |  |  |  |  |
|  |  | Conductivity: 0.0003 S/m per month |  |  |  |  |
|  |  | Temperature: 0.0002 °C per month |  |  |  |  |
|  |  | Pressure: Strain-gauge ± 0.1% of full-scale range; Quartz ± 0.02% of full-scale range per year |  |  |  |  |
|  |  | Resolution: - |  |  |  |  |
|  |  | Conductivity: 0.00005 S/m typical |  |  |  |  |
|  |  | Temperature: 0.0001 °C |  |  |  |  |
|  |  | Pressure: Strain-gauge 0.002% of full-scale range; Quartz 0.0025% of full-scale range |  |  |  |  |
|  |  | Sampling speed: Profiling: 4 Hz (strain-gauge pressure) or 2 Hz (Quartz pressure) |  |  |  |  |
|  |  | Memory & Data: Storage 64 Mbyte non-volatile FLASH Bytes/sample: 6 T&C; 5 pressures; 2 each external voltage; 4 date & time (RS-232 sensor is sensor dependent) |  |  |  |  |
|  |  | Power Supply & Consumption: 9 alkaline D-cell batteries, 60 hours CTD profiling (see manual) |  |  |  |  |
|  |  | Optional External Power: 9 - 28 VDC; consult factory for required current |  |  |  |  |
|  |  | Auxiliary Sensors Power: out up to 500 mA at 10.5 - 11 VDC; Voltage sensor A/D resolution 14 bits & input range 0-5 VDC |  |  |  |  |
|  |  | Housing, Depth Rating & Weight (add 0.3 to 0.7kg [in air] for pump, depending on model) Acetal Copolymer Plastic, 600 m, in air 7.3 kg, in water 2.3 kg 3AL-2.5V Titanium, 7000 m, in air 13.7 kg, in water 8.6 kg 6AL-4V Titanium, 10,500 m |  |  |  |  |
|  |  | Optional Cage (weight in air): (strain-gauge pressure version) 1016 x 241 x 279 mm, 6.3 kg, (Warranty: 3 years Minimum) |  |  |  |  |
| 16 | Acoustic Dopler Current Profiler | Features: Type 8382 (-0457 single battery; -0427 dual battery) | 1 |  |  |  |
|  |  | ADCP: Operating frequency - 625kHz |  |  |  |  |
|  |  | Maximum profiling range - 50+ m (depending on water environment) |  |  |  |  |
|  |  | Minimum cell size - 12mm |  |  |  |  |
|  |  | Minimum blanking distance - 0.6mm |  |  |  |  |
|  |  | Velocity range (along beam) - Up to ±2 m/s or 3.75 m/s user selectable |  |  |  |  |
|  |  | Velocity RMS - 0.5% of measured value |  |  |  |  |
|  |  | Maximum number of cells - 2500 |  |  |  |  |
|  |  | Maximum ping rate - 4Hz (5beams) |  |  |  |  |
|  |  | Beam width/ angle - ±1 degrees / 25 degrees |  |  |  |  |
|  |  | Acoustic modem: - Operating frequency - MF (20–34 kHz) |  |  |  |  |
|  |  | Typical operating range - 500m |  |  |  |  |
|  |  | Sensors: - Temperature - -5° to 35°C Heading accuracy/ resolution - ±1°/0.1° Pitch & roll accuracy/ resolution & range - ±1°/0.1° & ±90° (pitch), ±180° (roll) Pressure - ±0.05% full scale; inquire for options |  |  |  |  |
|  |  | Communication and Logging: - Communication - RS232, Ethernet and acoustic modem & Internal logging - 1 TB internal memory |  |  |  |  |
|  |  | Output: - Output telegrams - PD0, A gram, and B gram; simultaneous output |  |  |  |  |
|  |  | Electrical: - External power - 18-48 V power by external cable; PoE+, adapter included PoE+; adapter included Power - 15 mW (sleep), 600 mW (pinging), 3.5 W (fully active) Internal battery capacity - 55 Ah rechargeable (dual battery option available); charger included Internal battery recharge - 4 hours fast charge (8 hours for dual battery option) Full/ scheduled/ standby lifespan - 1 week/ 3 months/ 2 years |  |  |  |  |
|  |  | Environmental: - Depth rating - 300 m survivable/ 150 m operational Operating/ storage temperature: -5 to 40°C/ -20 to 55°C |  |  |  |  |
|  |  | Mechanical: - Construction - Pllastic Connector type - Subconn: 8-way for power and comms; 6-way for fast-charge Dimensions (height x max diameter) - 308 x 290 mm (dual battery height 388 mm) Weight in air/ water - 19.2/ 7.2 kg (dual battery option 23.9/ 7.9 kg) |  |  |  |  |
|  |  | Software: - Original Portal - Embedded Web UI for control & configuration Origin Scheduler - Schedule planning & configuration tool Origin Viewer - File data inspection Origin Topside - Remote configuration & control over acoustic modem, (Warrenty: 3 years Minimum) |  |  |  |  |
| 17 | Tide Gauge (Non-Contact Radar System) | Features: - Contactless water level measurement | 1 |  |  |  |
|  |  | Sensors: - RS-232, RS-485 Modbus, SDI-12, analog 4-20 mA, Robust, small-size IP68 enclosure |  |  |  |  |
|  |  | Data Logger - handle harsh weather, floods, high winds and rough seas |  |  |  |  |
|  |  | Operating temperature- range from -40°C to 70°C |  |  |  |  |
|  |  | Design for use with data loggers |  |  |  |  |
|  |  | Adjustable angle - 30 and 45 degrees |  |  |  |  |
|  |  | weather tight |  |  |  |  |
|  |  | Water Level Temperature Sensors - Vented or absolute pressure sensor options with 0.05% full scale accuracy |  |  |  |  |
|  |  | SDI-12, RS-485 Modbus RTU, and 4-20mA protocols for interfacing with data loggers and PLC systems |  |  |  |  |
|  |  | Mounting kit, (Warranty: 3 years Minimum) |  |  |  |  |
| 18 | Spectrophotometer | Make – Please Specify | 1 |  |  |  |
|  |  | Model – Please Specify |  |  |  |  |
|  |  | Country of Origin – USA/Europe/Japan |  |  |  |  |
|  |  | Warranty – 01 Year or Specify |  |  |  |  |
|  |  | Optical System – Reference Beam, Spectral |  |  |  |  |
|  |  | Light Source – Gas-filled Tungsten |  |  |  |  |
|  |  | Wavelength Range – 330–1000 nm or better |  |  |  |  |
|  |  | Wavelength Accuracy – ±1.5 nm or better |  |  |  |  |
|  |  | Wavelength Calibration – Automatic |  |  |  |  |
|  |  | Spectral Bandwidth – 5 nm or better |  |  |  |  |
|  |  | Photometric Measuring Range – ±3.0 Abs or Better |  |  |  |  |
|  |  | Pre-programmed Methods – 200 or better |  |  |  |  |
|  |  | Reagent Recognition – RFID & Barcode |  |  |  |  |
|  |  | Display – 5.5" Color or Better |  |  |  |  |
|  |  | Data Storage – 1500 Data Points or Better |  |  |  |  |
|  |  | Enclosure Rating – IP30 or Better |  |  |  |  |
|  |  | Interface – USB (2), Ethernet and RFID |  |  |  |  |
|  |  | Data Logger – 2000 Measured Values |  |  |  |  |
|  |  | Sample Cell Compatibility – 10 & 50 mm Rectangular, 13 mm Round, 1 Round, 1 Square (Warranty: 3 years Minimum) |  |  |  |  |
| 19 | Oven | 7 sizes 6 to 100 liters | 1 |  |  |  |
|  |  | Temp range 40 °C to 250 °C |  |  |  |  |
|  |  | Fluctuation +/- 0.75 °C |  |  |  |  |
|  |  | Easy to use thermostatic control |  |  |  |  |
|  |  | Independent overheat thermostat (Warranty: 3 years Minimum) |  |  |  |  |
| 20 | Muffle Furnaces | Tmax 1100 °C or 1200 °C | 1 |  |  |  |
|  |  | Heating from two sides by ceramic heating plates Thermocouple type N (1100 °C) or type S (1200 °C) Ceramic heating plates with integral heating element which is safeguarded and easy to replace Optional flap door (L) which can be used as work platform or lift door (LT) with hot surface facing away from the operator Adjustable air inlet integrated in door (see illustration) · Exhaust air outlet in rear wall of furnace Controller with touch operation B510 (Warranty: 3 years Minimum) |  |  |  |  |
| 21 | Distilled Water Plant | Compact and user-friendly tabletop design. | 1 |  |  |  |
|  |  | 4L/hr. Capacity |  |  |  |  |
|  |  | Should be supplied a thermometer for displaying the cooling water temperature. |  |  |  |  |
|  |  | Easily openable Boiling Chamber as well as Condenser for easy cleaning. |  |  |  |  |
|  |  | Heating power - 3kW |  |  |  |  |
|  |  | Degas Function for remove CO2 via condenser vent. |  |  |  |  |
|  |  | Safety features - Thermostatic cut-off (self -resetting type) for assured low water detection - with visual indication and safety alarm. |  |  |  |  |
|  |  | Power supply AC 220-240V, 50/60 Hz |  |  |  |  |
|  |  | Spare parts availability certificate for at least 10 years by the Manufacture. |  |  |  |  |
|  |  | Warranty – Minimum 1 year |  |  |  |  |
|  |  | Local Agent authorization letters. |  |  |  |  |
|  |  | User reference list. |  |  |  |  |
|  |  | Certification – CE and ISO standard |  |  |  |  |
| 22 | Multiparameter | Measures DO, pH, ORP, conductivity, salinity, TDS, turbidity, temperature, depth, and optional nutrients (ammonium, nitrate, chloride), plus algae/chlorophyll., Waterproof (IP-67 rated), Stores 100,000+ datasets, supports USB data transfer (Warranty: 3 years (Minimum) | 1 |  |  |  |
| 23 | Salinometer | Measurement range – Typically 0–70 ppt salinity (practical salinity units), suitable for freshwater to seawater, Accuracy – High precision with ±1% of reading or better, Measurement principle – Based on electrical conductivity using a multi-electrode sensor., Handheld, water-resistant/waterproof (Warranty: 3 years (Minimum) | 1 |  |  |  |
| 24 | Soxhlet Extraction Unit | Positions: 6; Extraction Flasks: 250–500 ml borosilicate glass; Condenser Type: Vertical glass condenser; Heating Mantle: Individual, digital temperature control 50–300 °C; Material: Heat-resistant glass, stainless steel supports; Power: 220 V, 50 Hz; Accessories: Clamps, connectors, silicone tubing; Max Solvent Volume per unit: 500 ml (Warranty: 3 years (Minimum) | 1 |  |  |  |
| 25 | Sedwick Rafter | Overall, Glass Dimensions | 30 |  |  |  |
|  |  | 76mm x 40mm x 2.28mm thick |  |  |  |  |
|  |  | Chamber Dimensions |  |  |  |  |
|  |  | 50mm x 20mm x 1mm deep |  |  |  |  |
|  |  | Volume 1ml |  |  |  |  |
| 26 | Food Processor | Key Features | 1 |  |  |  |
|  |  | 700 W |  |  |  |  |
|  |  | 16 functions |  |  |  |  |
|  |  | 2-in-1 disc |  |  |  |  |
|  |  | In-bowl storage |  |  |  |  |
|  |  | (Warranty: 3 years (Minimum) |  |  |  |  |
| 27 | Gas Cylinder (5Kg) | Litro | 1 |  |  |  |
| 28 | Light Microscope | Material: Alloy | 1 |  |  |  |
|  |  | Draw Tube: Monocular |  |  |  |  |
|  |  | Mechanical tube length: 120 mm |  |  |  |  |
|  |  | Eyepiece: 10x |  |  |  |  |
|  |  | Objective: 4x,10x,40x |  |  |  |  |
|  |  | Total Magnification: 40X-400 |  |  |  |  |
|  |  | Stage: Single layer with paired slid clips |  |  |  |  |
|  |  | Stage Size: 90*90mm |  |  |  |  |
|  |  | Diaphragm: Rotating disc diaphragm |  |  |  |  |
|  |  | Nosepiece: Triple |  |  |  |  |
|  |  | Focusing: Coarse adjustment focusing only. (Warranty: 3 years (Minimum) |  |  |  |  |
| 29 | Spectrophotometer | CertiRef | 1 |  |  |  |
|  |  | CuvetteChanger |  |  |  |  |
|  |  | FillPalMini |  |  |  |  |
|  |  | Handheld barcode scanner |  |  |  |  |
|  |  | Immersion |  |  |  |  |
|  |  | LabX PC Software |  |  |  |  |
|  |  | LinSet |  |  |  |  |
|  |  | Printer USB-P25 (Warranty: 3 years (Minimum) |  |  |  |  |
| 30 | Muffle Furnaces | Tmax 1100 °C or 1200 °C | 1 |  |  |  |
|  |  | Heating from two sides by ceramic heating plates |  |  |  |  |
|  |  | Thermocouple type N (1100 °C) or type S (1200 °C) |  |  |  |  |
|  |  | Ceramic heating plates with integral heating element which is safeguarded and easy to replace |  |  |  |  |
|  |  | Optional flap door (L) which can be used as work platform or lift door (LT) with hot surface facing away from the operator |  |  |  |  |
|  |  | Adjustable air inlet integrated in door (see illustration) · Exhaust air outlet in rear wall of furnace |  |  |  |  |
|  |  | Controller with touch operation B510 (Warranty: 3 years Minimum) |  |  |  |  |
| 31 | Oven | 7 sizes 6 to 100 liters | 1 |  |  |  |
|  |  | Temp range 40 °C to 250 °C |  |  |  |  |
|  |  | Fluctuation +/- 0.75 °C |  |  |  |  |
|  |  | Easy to use thermostatic control |  |  |  |  |
|  |  | Independent overheat thermostat (Warranty: 3 years Minimum) |  |  |  |  |
| 32 | CHN Elemental Analyzer | Measuring Range: Carbon: 0.005% to 100%, Hydrogen: 0.01% to 60%, Nitrogen: 0.008% to 60% | 1 |  |  |  |
|  |  | Repeatability: Carbon (Cad ≤ 0.5%), Hydrogen (Had ≤ 0.15%), Nitrogen (Nad ≤0.08%) |  |  |  |  |
|  |  | Temperature Range: RT+10 to 1050℃ |  |  |  |  |
|  |  | Temperature Accuracy: ±1℃ |  |  |  |  |
|  |  | Sample Weight: 80 to 100mg |  |  |  |  |
|  |  | Sample Delivery Method: Automatic |  |  |  |  |
|  |  | Number Of Samples: 39 Samples/Time |  |  |  |  |
|  |  | Single Sample Analysis Time: Around 4min |  |  |  |  |
|  |  | Carrier Gas: Helium, Purity ≥99.995%, Pressure 0.25±0.1mpa |  |  |  |  |
|  |  | Supporting Gas: Oxygen, Purity for Hydrocarbons ≥99.5%, Purity for Nitrogen Element ≥99.995%, Pressure |  |  |  |  |
|  |  | Power Gas: Inert Gas, No Oil, And No Water, Pressure 0.29±0.1mpa |  |  |  |  |
|  |  | Placement Method: Horizontal |  |  |  |  |
|  |  | Power: ≤4.5kw |  |  |  |  |
|  |  | Power Supply: 220V 50-60Hz |  |  |  |  |
|  |  | Dimension: 690 × 740 × 720 mm |  |  |  |  |
|  |  | Packaging Dimension: 700 × 750 × 720 mm |  |  |  |  |
|  |  | Net Weight: 102 Kg |  |  |  |  |
|  |  | Gross Weight: 105 Kg (Warranty: 3 years Minimum) |  |  |  |  |
| 33 | Water Distiller | Compact and user-friendly tabletop design. | 1 |  |  |  |
|  |  | 4L/hr. Capacity |  |  |  |  |
|  |  | Should be supplied a thermometer for displaying the cooling water temperature. |  |  |  |  |
|  |  | Easily openable Boiling Chamber as well as Condenser for easy cleaning. |  |  |  |  |
|  |  | Heating power - 3kW |  |  |  |  |
|  |  | Power supply AC 220-240V, 50/60 Hz |  |  |  |  |
|  |  | Degas Function for remove CO2 via condenser vent. |  |  |  |  |
|  |  | Safety features - Thermostatic cut-off (self -resetting type) for assured low water detection - with visual indication and safety alarm. |  |  |  |  |
|  |  | Power supply AC 220-240V, 50/60 Hz |  |  |  |  |
|  |  | Spare parts availability certificate for at least 10 years by the Manufacture. |  |  |  |  |
|  |  | Warranty – Minimum 1 year |  |  |  |  |
|  |  | Local Agent authorization letters. |  |  |  |  |
|  |  | User reference list. |  |  |  |  |
|  |  | Certification – CE and ISO Standard |  |  |  |  |
| 34 | Orbital Shaker | Voltage(V) 220-240 Frequency(Hz) 50~60 Motor Type - DC brushless motor Max Load(kg) 7.5 Allowable Ambient Temperature 5~40℃ Allowable Ambient Humidity 80% Speed Range(rpm) 60~500 LCD Speed Display Timing Function Availability Timing Range should be 0~99h59min LCD Time Display Heating power 35/185W Amplitude 4mm Orbital Shaking mode Temperature Range RT+5~75℃ Temperature Accuracy 0.1 ℃ Instrument Size(mm) 330*370*130 (Approx.) Warranty minimum 1 year Spare parts availability (Specify) Local Agent authorization letters User reference list CE Certified | 1 |  |  |  |
| 35 | Freezer | Total Capacity=320 Liters Shelves=Wire baskets Cooling Method=Direct cooling Refrigerant=R600a (Eco-friendly) Compressor=Conventional Compressor Temperature Control=Mechanical Adjustable Thermostat Voltage/Frequency =220-240V AC / 50Hz Net Weight=Approx. 45 kg Material=Coated metal body + aluminum evaporator Defrost=Manual Defrost Lighting=Interior Light Alarms needed for Over-temperature alarm Outer Doors and locks Single lid + door lock Warranty Period 1 Years Manufacturer Warranty 5 Years Compressor Warranty Additional Features Compressor indicator Drain system for cleaning Easy-moving castors | 1 |  |  |  |
| 36 | Desiccator | Material: Made of high-quality Borosilicate Glass 3.3 (SIMAX or equivalent), ensuring durability and compliance with international laboratory standards. | 1 |  |  |  |
|  |  | Functionality: |  |  |  |  |
|  |  | Suitable for drying and dehydration processes of laboratory samples. |  |  |  |  |
|  |  | When equipped with a stopcock/valve, the desiccator shall be suitable for vacuum applications and storage under reduced pressure. |  |  |  |  |
|  |  | Chemical Resistance: |  |  |  |  |
|  |  | Shall exhibit excellent resistance to a wide range of chemicals, ensuring long-term usability without degradation. |  |  |  |  |
|  |  | Thermal Properties: |  |  |  |  |
|  |  | Low coefficient of thermal expansion |  |  |  |  |
|  |  | High resistance to thermal shock, minimizing risk of cracking during temperature variations |  |  |  |  |
|  |  | Sterilization: |  |  |  |  |
|  |  | Must be autoclavable, allowing safe sterilization under high temperature and pressure conditions. |  |  |  |  |
|  |  | Design & Construction: |  |  |  |  |
|  |  | Provided with a ground flat flange for airtight sealing |  |  |  |  |
|  |  | Precision-ground surfaces to ensure leak-proof performance, especially under vacuum conditions |  |  |  |  |
|  |  | Standards & Certification: |  |  |  |  |
|  |  | Manufactured in compliance with relevant DIN standards for laboratory glassware |  |  |  |  |
|  |  | Product shall comply with Marienfeld Superior brand certifications and ISO-certified manufacturing processes |  |  |  |  |
|  |  | Diameter=300mm Should be supplied with disk (Warranty: 3 years Minimum) |  |  |  |  |
| 37 | Laboratory Scale 1 | Capacity- 2200 g or better | 1 |  |  |  |
|  |  | Minimum loading – 0.5 g |  |  |  |  |
|  |  | Readability- (d) 0.01 g |  |  |  |  |
|  |  | Repeatability – 0.01 g |  |  |  |  |
|  |  | Linearity +/- 0.02 g |  |  |  |  |
|  |  | Response Time 2 sec. or better |  |  |  |  |
|  |  | Minimum Sample weight - 1 g |  |  |  |  |
|  |  | MonoTech weighting sensor based on electromagnetic force for high, reliable accuracy and long operation life. |  |  |  |  |
|  |  | self-calibration according to temperature changes or pre-set conditions. |  |  |  |  |
|  |  | Ability to configure perfect self-automatic calibration (PSAC) to initiate self-calibration at specific time and every power on. |  |  |  |  |
|  |  | Standard USB to connect printer and R232 interface to windows direct communication with windows applications. |  |  |  |  |
|  |  | Balances have adjustable feet for maintaining/providing uniform level. |  |  |  |  |
|  |  | Statistic printing to obtain data including minimum value, Standard deviation, difference and coefficient variance. |  |  |  |  |
|  |  | Weighing modes of weight measurement, Piece Counting, animal weighing mode, Percentage weighing and density measurement. |  |  |  |  |
|  |  | Pan size should(mm) 190 x 205 with stainless steel pan |  |  |  |  |
|  |  | Solid construction with high quality material. |  |  |  |  |
|  |  | Dimensions (W x D) 345 x 215 mm or less |  |  |  |  |
|  |  | Tare rage - full |  |  |  |  |
|  |  | Built in features for ensures GLP/ISO regulation compliance. |  |  |  |  |
|  |  | Back-lit function for easy viewing. |  |  |  |  |
|  |  | Operating temperature range should be 15 C - 35 C. |  |  |  |  |
|  |  | Power supply 220-230V 50 -60 Hz |  |  |  |  |
|  |  | Please offer optional prices for a rechargeable battery. |  |  |  |  |
|  |  | Ability to attend within 2 days for any service maintenance or calibration needs. |  |  |  |  |
|  |  | All technical data in required specifications should be mentioned and highlighted clearly in product brochure. |  |  |  |  |
|  |  | Product manual with technical data sheets and product demonstrations should be provided with the product. |  |  |  |  |
|  |  | Warranty – Minimum 2 Years. |  |  |  |  |
|  |  | Spare parts availability- Please specify. |  |  |  |  |
|  |  | Local Agent authorization letters. |  |  |  |  |
|  |  | List of users for Identical model |  |  |  |  |
| 38 | Laboratory Scale 2 | Capacity- 6100 g or better | 1 |  |  |  |
|  |  | Readability- (d) 0.01 g |  |  |  |  |
|  |  | Repeatability – 0.02 g |  |  |  |  |
|  |  | Linearity +/- 0.03 g |  |  |  |  |
|  |  | Response Time 2 sec. or better |  |  |  |  |
|  |  | Minimum Sample weight - 1 g |  |  |  |  |
|  |  | MonoTech weighting sensor based on electromagnetic force for high, reliable accuracy and long operation life. |  |  |  |  |
|  |  | self-calibration according to temperature changes or pre-set conditions. |  |  |  |  |
|  |  | Ability to configure perfect self-automatic calibration (PSAC) to initiate self-calibration at specific time and every power on. |  |  |  |  |
|  |  | Standard USB to connect printer and R232 interface to windows direct communication with windows applications. |  |  |  |  |
|  |  | Balances have adjustable feet for maintaining/providing uniform level. |  |  |  |  |
|  |  | Statistic printing to obtain data including minimum value, Standard deviation, difference and coefficient variance. |  |  |  |  |
|  |  | Weighing modes of weight measurement, Piece Counting, animal weighing mode, Percentage weighing and density measurement. |  |  |  |  |
|  |  | Pan size should(mm) 190 x 205 with stainless steel pan |  |  |  |  |
|  |  | Solid construction with high quality material. |  |  |  |  |
|  |  | Dimensions (W x D) 345 x 215 mm or less |  |  |  |  |
|  |  | Tare rage - full |  |  |  |  |
|  |  | Built in features for ensures GLP/ISO regulation compliance. |  |  |  |  |
|  |  | Back-lit function for easy viewing. |  |  |  |  |
|  |  | Operating temperature range should be 15 C - 35 C. |  |  |  |  |
|  |  | Power supply 220-230V 50 -60 Hz |  |  |  |  |
|  |  | Please offer optional prices for a rechargeable battery. |  |  |  |  |
|  |  | Ability to attend within 2 days for any service maintenance or calibration needs. |  |  |  |  |
|  |  | All technical data in required specifications should be mentioned and highlighted clearly in product brochure. |  |  |  |  |
|  |  | Product manual with technical data sheets and product demonstrations should be provided with the product. |  |  |  |  |
|  |  | Warranty – Minimum 2 Years. |  |  |  |  |
|  |  | Spare parts availability- Please specify. |  |  |  |  |
|  |  | Local Agent authorization letters. |  |  |  |  |
|  |  | List of users for Identical model |  |  |  |  |
| 39 | Trinocular Microscope with 8MP Camera | Observation method=Brightfield | 1 |  |  |  |
|  |  | Optical system=Infinity Corrected Optical System |  |  |  |  |
|  |  | Head=Trinocular, 30/360 rotating |  |  |  |  |
|  |  | Split Ratio=50/50 |  |  |  |  |
|  |  | Interpupillary distance (mm)=50-75 |  |  |  |  |
|  |  | Dioptric adjustment=On left tube |  |  |  |  |
|  |  | Eyepieces=10x/22 mm, Higher eye point with fixing screws |  |  |  |  |
|  |  | Nosepiece=Quintuple and Bi-directional |  |  |  |  |
|  |  | Objective type=IOS W-PLAN |  |  |  |  |
|  |  | Parfocal distance (mm)=45 |  |  |  |  |
|  |  | Standard objectives |  |  |  |  |
|  |  | 4x/0.10, W.D. 17.3 mm |  |  |  |  |
|  |  | 10x/0.25, W.D. 10 mm |  |  |  |  |
|  |  | 40x/0.65, W.D. 0.54 mm |  |  |  |  |
|  |  | 100x/1.25 (Oil), W.D. 0.13 mm |  |  |  |  |
|  |  | Focusing System |  |  |  |  |
|  |  | Coaxial coarse and fine with focus memory lock |  |  |  |  |
|  |  | Fine resolution (μm) =2 |  |  |  |  |
|  |  | Fine total travel (per single rotation) (mm)=0.2 |  |  |  |  |
|  |  | Fine graduations=100 |  |  |  |  |
|  |  | Coarse total travel (mm)=25 Stage Type=Double layer, rackless Stage Dimensions=233x147 Stage XY Moving range (mm)=78x54 Vernier scale accuracy (mm)=0.1 Condenser Type=Swing-out, Centrale and removable Abbe condenser with focusable by rack and pinion with aperture scale Condenser Numerical aperture (N.A.) 0.2/0.9 Transmitted Illumination=Full Kohler illumination X-LED With manual light control facility LED life time=Long-life more than 65,000 hours Light source power (W)=3.6 Power Supply=External Multi-plug Warranty=10 years Spare parts availability=10 years Local Agent authorization letters=Yes User reference list=More than 10 years - USB Camera resolution (MP)=8.3 - 2. Camera resolution (n° of pixels: W x H) =3840x2160 - Color / Monochrome=Color - Sensor Size=1/1.8" - Sensor Technology=CMOS - Sensor Type=SONY EXMOR - Acquisition technology =Rolling Shutter - Pixel Size (um)=2.0 x 2.0 - Frame rate full resolution (fps)=45@3840x2160 - Frame rate other resolution (fps)=70@1920x1080 - Exposure time=0.02 msec - 15 sec - IR filter=Should available - C-Mount=Should be supplied with compatible C Mount adaptor - Camera accessories Calibration slide USB cable (1.8m) - Camera Software Should be supplied free of charge from the same manufacturer Image analyzing software Compatible with Windows 7, Windows 8, Windows 10 (32Bit), Windows 10 (64Bit), Osx (MAC Osx) starting from 10.7.5, Linux - Warranty 02 years - Spare parts availability = 02 years - Local Agent authorization letters = Yes - User reference list, more than 5 years - Laptop Laptop, 8 GB Ram, 11th Gen |  |  |  |  |
| 40 | Laboratory Oven | ISO 13485, ISO 9001: 2015 and CE certifications. – Relevant document should be attached. | 1 |  |  |  |
|  |  | PED certified manufacture by a European body. (PED -2014 / 68 /EU) |  |  |  |  |
|  |  | Product should comply with all the essential health and safety requirements (EC Declaration of conformity) - EN ISO 12100 :2010, EN ISO 14119 :2013, EN2006/42/EC, 2014/35/EU, 2014/30/EU. – Relevant documents should be attached. |  |  |  |  |
|  |  | Capacity (L) =Not less than 40 L |  |  |  |  |
|  |  | Temperature range=50 to 200 ℃ |  |  |  |  |
|  |  | Temperature control accuracy=±2 ℃ |  |  |  |  |
|  |  | Inner Dimensions Maximum. (WxDxH)=350 x 400 x 400 mm |  |  |  |  |
|  |  | Outer dimensions Maximum. (WxDxH)=600 x 500 x 800 mm |  |  |  |  |
|  |  | Shelves=2 nos. |  |  |  |  |
|  |  | Heater power (kW)=Maximum 1 kW |  |  |  |  |
|  |  | Power supply =230VAC, 50Hz, Single phase. (Warranty: 3 years Minimum) |  |  |  |  |
| 41 | Student Microscope | Observation method=Brightfield | 2 |  |  |  |
|  |  | Optical system=Infinity corrected |  |  |  |  |
|  |  | Head=Binocular, 360 rotating |  |  |  |  |
|  |  | Inclination=30° |  |  |  |  |
|  |  | Interpupillary distance (mm)=48-75 |  |  |  |  |
|  |  | Dioptric adjustment=Should be on both tubes |  |  |  |  |
|  |  | Eyepieces |  |  |  |  |
|  |  | 10x, Higher eye point with fixing screws |  |  |  |  |
|  |  | Field Number=20mm |  |  |  |  |
|  |  | High eyepoint (For glass wearers) Should be available |  |  |  |  |
|  |  | Nosepiece=Quadruple, Reversed, Bi-Directional |  |  |  |  |
|  |  | Objective type=PLAN Achromatic |  |  |  |  |
|  |  | Objectives=Should be anti-fungus treated |  |  |  |  |
|  |  | Standard Magnification=40X to 1000X |  |  |  |  |
|  |  | Standard objectives |  |  |  |  |
|  |  | 4x/0.10, W.D. 16.8 mm |  |  |  |  |
|  |  | 10x/0.25, W.D. 5.8 mm |  |  |  |  |
|  |  | 40x/0.65, W.D. 0.43 mm |  |  |  |  |
|  |  | 100x/1.25 (Oil), W.D. 0.13 mm |  |  |  |  |
|  |  | 100x/1.25 (Water), W.D. 0.13 mm |  |  |  |  |
|  |  | Focusing System=Coaxial and Fine |  |  |  |  |
|  |  | Fine resolution (μm) and Coarse total travel (mm) 4 μm, 18mm at least |  |  |  |  |
|  |  | Stage Dimensions At least 150x135 |  |  |  |  |
|  |  | Stage XY Moving range (mm) 75x30 |  |  |  |  |
|  |  | Condenser Type |  |  |  |  |
|  |  | Single Position Centrable Abbe condenser with focusable by rack and pinion and 1.25NA |  |  |  |  |
|  |  | Light source and power (W) X-LED3, 3.6W |  |  |  |  |
|  |  | Lifetime of Power source= More than 65,000hours |  |  |  |  |
|  |  | Power Supply =External Multi-plug |  |  |  |  |
|  |  | Should include |  |  |  |  |
|  |  | · Dust cover |  |  |  |  |
|  |  | · Immersion oil-10ml |  |  |  |  |
|  |  | · Tension adjustment tool |  |  |  |  |
|  |  | · Microscope Slide packs(72/Pk) *02 |  |  |  |  |
|  |  | Warranty |  |  |  |  |
|  |  | · At least 10 years for microscope optical and mechanical parts |  |  |  |  |
|  |  | Spare parts availability= 10 years |  |  |  |  |
|  |  | Local Agent authorization letters =Yes |  |  |  |  |
|  |  | User reference list |  |  |  |  |
|  |  | More than 5 years |  |  |  |  |
| 42 | Crucibles | Material: Made of high-quality glazed porcelain, suitable for high-temperature laboratory applications and ensuring excellent mechanical strength. | 40 |  |  |  |
|  |  | Functionality: Suitable for heating, ignition, and ashing of samples in laboratory procedures, including use in furnaces. |  |  |  |  |
|  |  | Chemical Resistance: Shall exhibit excellent resistance to acids, alkalis, and chemical reagents, ensuring stability during analytical processes. |  |  |  |  |
|  |  | Thermal Properties: Minimal thermal expansion |  |  |  |  |
|  |  | High resistance to thermal shock, reducing risk of cracking under rapid heating and cooling cycles |  |  |  |  |
|  |  | Temperature Resistance: |  |  |  |  |
|  |  | Surface Finish: Fully glazed surface, providing: |  |  |  |  |
|  |  | Smooth and non-porous finish |  |  |  |  |
|  |  | Easy cleaning and reduced contamination |  |  |  |  |
|  |  | Enhanced chemical resistance |  |  |  |  |
|  |  | Design & Construction: |  |  |  |  |
|  |  | Uniform wall thickness for even heat distribution |  |  |  |  |
|  |  | Stable base and proper shape for safe handling and furnace use |  |  |  |  |
|  |  | Optional lids (if required) shall ensure proper fitting |  |  |  |  |
|  |  | Standards & Certification: |  |  |  |  |
|  |  | Manufactured in compliance with relevant DIN standards for laboratory porcelain ware |  |  |  |  |
|  |  | Produced under ISO-certified manufacturing processes |  |  |  |  |
|  |  | Shall comply with Marienfeld Superior brand quality certifications |  |  |  |  |
|  |  | Diameter = 30mm |  |  |  |  |
|  |  | height= 35- 40mm |  |  |  |  |
|  |  | capacity= 15ml |  |  |  |  |
| 43 | Petri Dish | Autoclavable glass | 100 |  |  |  |
|  |  | heat up to 200-degree Celsius |  |  |  |  |
|  |  | 100X15MM |  |  |  |  |
|  |  | Operating temperature range should be 15 C - 35 C. |  |  |  |  |
| 44 | Fume hood | Overall dimensions should be approximately (WxDxH) 1200x 850x 2350mm | 1 |  |  |  |
|  |  | Work chamber dimensions should be approximately (WxDxH) 960x 620x 1180mm |  |  |  |  |
|  |  | Cabinet outer surfaces should be made of high-quality 8mm porcelain white polypropylene |  |  |  |  |
|  |  | Inner working chamber should be high quality 8mm porcelain white polypropylene |  |  |  |  |
|  |  | Front Window Resistant to acid and alkali, Manual, 5mm toughened glass, height adjustable |  |  |  |  |
|  |  | Material Main Body – Dual wall construction made of Porcelain white PP, thickness 8 mm, integrally welded resistant to strong acid, alkali and anti-corrosion. |  |  |  |  |
|  |  | Should be included chemical storage base cabinet with 3 doors with 1 shelf, Base cabinet construction material should be anti-corrosion polypropylene |  |  |  |  |
|  |  | Air Volume (m³/h) @ 100% sash open, 1500-2000m³/h |  |  |  |  |
|  |  | Air velocity should be 0.3~0.8m/s |  |  |  |  |
|  |  | Exhaust duct opening diameter is 250mm |  |  |  |  |
|  |  | Should have a Built-in PP anti-corrosion centrifugal fan, rpm is 1400rpm, air volume 2000-2300m³/h, wind pressure 30-28 mmHg. |  |  |  |  |
|  |  | Max Opening Height 500mm |  |  |  |  |
|  |  | Noise ≤60dB |  |  |  |  |
|  |  | LED lamp: 1 unit of 24W, 6000K white light lamp |  |  |  |  |
|  |  | Power Supply AC 220V, 50/60Hz |  |  |  |  |
|  |  | Work surface should be chemical resistant epoxy resin 19mm |  |  |  |  |
|  |  | Standard Accessory - LED light, Base cabinet, 10A socket x 4, Water tap (PP), Gas tap (PP), Gas & Water remote control, Water Sink |  |  |  |  |
|  |  | Microprocessor control system, with air volume is controlled by damper valve system and LED display |  |  |  |  |
|  |  | Ducting with PVC 225mm pipe with appropriate blower and bends should be provided and installed by the supplier. |  |  |  |  |
|  |  | Bidders must support compliance to the given specifications with manufacturer's original brochures |  |  |  |  |
|  |  | The equipment shall be warranted for period of not less than 24 calendar months from the date of successful commissioning on a full "parts & labor" replacement basis. The tenderer shall specify in detail the servicing and repair facilities and means available to them both (locally and from manufacturers) to implement such a warranty without under delay in Sri Lanka. |  |  |  |  |
|  |  | Bidders must support compliance to the given specifications with manufacturer's original brochures |  |  |  |  |
|  |  | Bidders must provide documentary evidence on their technical capacity (trained staff, testing equipment for calibration & maintenance support as per manufacturer service/technical manuals) to carry out installation and service. |  |  |  |  |
|  |  | Warranty for 2 years period for the whole system from the date of installation and commissioning the equipment. |  |  |  |  |
|  |  | Free regular preventive service maintenance should be provided during the warranty period with free labor and parts. |  |  |  |  |
|  |  | Specify the details of preventive maintenance and any calibration procedures needed after 2 years warranty period. Specify the details of the cost for annual maintenance service and after the warranty period. |  |  |  |  |
|  |  | Installation and onsite training should be provided free of charge |  |  |  |  |
|  |  | Local agent must ensure repairing of machine in sudden out of order situation within three days. Any cost of service arranged at private sector due to delays in repair more than 3 days, should be borne by the local agent |  |  |  |  |
|  |  | There should be quality after sale service. Provide details. |  |  |  |  |
|  |  | Provide the letter of authorization from the manufacturer |  |  |  |  |
|  |  | Spare parts of the system should be available at least for a period of 5 years after installation and provide the fixed prices of spare parts of the system for a period of 5 years. |  |  |  |  |
|  |  | Operational conditions (temperature and humidity) of the machine should be compatible with local settings. Provide the details |  |  |  |  |
| 45 | Glass Spreader | 145x59x5mm | 10 |  |  |  |
|  |  | spreading bacteria culture on solid agar media |  |  |  |  |
| 46 | Dissection Tray | Aluminum tray with wax, size: 300 x 200 x 45 mm | 30 |  |  |  |
| 47 | Refrigerator | Total Capacity=250 Liters | 1 |  |  |  |
|  |  | Shelves= Spill-proof adjustable shelves |  |  |  |  |
|  |  | Cooling Method=No Frost |  |  |  |  |
|  |  | Refrigerant=R600a (Eco-friendly) |  |  |  |  |
|  |  | Compressor |  |  |  |  |
|  |  | Inverter Technology |  |  |  |  |
|  |  | Temperature Control |  |  |  |  |
|  |  | Mechanical Thermostat |  |  |  |  |
|  |  | Voltage/Frequency =220-240V AC / 50Hz |  |  |  |  |
|  |  | Product Dimensions (H x W x D) =Approx. 1480 x 540 x 712 mm |  |  |  |  |
|  |  | Packaging Dimensions (H x W x D) =Approx. 1550 x 585 x 755 mm |  |  |  |  |
|  |  | Net Weight=Approx. 51.5 kg |  |  |  |  |
|  |  | Gross Weight=Approx. 56.3 kg |  |  |  |  |
|  |  | Insulation=2-Inch-thick thermal insulated wall |  |  |  |  |
|  |  | Material=multi-layer coated cabinets and doors |  |  |  |  |
|  |  | Tubing =Copper tubing for efficient cooling |  |  |  |  |
|  |  | Defrost=No Frost system |  |  |  |  |
|  |  | Lighting=Bright and long-lasting LED light |  |  |  |  |
|  |  | Outer Doors and locks |  |  |  |  |
|  |  | Double door + door lock |  |  |  |  |
|  |  | Warranty Period |  |  |  |  |
|  |  | 2 Years Manufacturer Warranty |  |  |  |  |
|  |  | 5 Years Body Corrosion Warranty |  |  |  |  |
|  |  | 10 Years Compressor Warranty |  |  |  |  |
| 48 | Mixer Grinder | Bench-top laboratory grinder / mini pulverize | 1 |  |  |  |
|  |  | Suitable for dry powder preparation |  |  |  |  |
|  |  | Capacity 1 Batch capacity: 50 g – 500 g per cycle |  |  |  |  |
|  |  | Grinding Performance |  |  |  |  |
|  |  | Final particle size: ~75 µm to fine powder |  |  |  |  |
|  |  | Adjustable fineness 3 Uniform particle size distribution |  |  |  |  |
|  |  | Motor & Power 1 Motor power: 0.75 – 1 HP (≈ 500 – 750 W) |  |  |  |  |
|  |  | Power supply: 220–240 V, 50 Hz 5. Construction |  |  |  |  |
|  |  | Grinding chamber: Stainless Steel (SS 304 / SS 316) |  |  |  |  |
|  |  | Disc / blade type grinding 3 Corrosion-resistant body 6. Feed & Output 1 Max feed size: ≤ 5–6 mm 2 Output: ≤ 100 mesh powder |  |  |  |  |
|  |  | Safety Features = Lid safety interlock 2 Overload / thermal protection |  |  |  |  |
|  |  | Operation & Control |  |  |  |  |
|  |  | Simple ON/OFF switch |  |  |  |  |
|  |  | Optional timer 3 Adjustable sieve / grinding gap |  |  |  |  |
|  |  | Dimensions & Weight 1 Approx size: 50 × 30 × 30 cm 2 Weight: 15 – 35 kg |  |  |  |  |
|  |  | Applications = Soil, plant, biological samples, Chemicals and food analysis |  |  |  |  |
|  |  | Cleaning & Maintenance |  |  |  |  |
|  |  | Easy to dismantle, Washable chamber |  |  |  |  |
|  |  | Warranty 1 Minimum 1 year warranty 2 Local service support |  |  |  |  |